Assignment – 3

- Common Issues with USB Keyboard or Mouse:

| No. | Common issue | Possible Solution |
| --- | --- | --- |
| 1 | Keyboard/mouse is not detected | Unplug and reconnect it, or try another USB port. |
| 2 | Keyboard or mouse is not responding | Restart the laptop and reconnect the device. |
| 3 | Some keys/buttons are not working | Clean the device and check for physical damage. |
| 4 | Device works intermittently | Try a different USB port or check for a loose USB connection. |
| 5 | Driver problem | Open Device Manager and update or reinstall the keyboard/mouse driver. |

- Bluetooth Headphones Not Connecting to Phone:

Troubleshooting Flow:

- Check Bluetooth: Open Settings → Bluetooth and make sure Bluetooth is ON.

- Check headphones: Make sure the headphones are powered ON and have enough battery.

- Enter pairing mode: Press and hold the Bluetooth/power button until the indicator light starts blinking.

- Check available devices: Look for the headphone name under Available Devices.

- Pair the headphones: Tap the headphone name and select Pair/Connect.

- Check distance: Keep the headphones close to the phone and remove obstacles.

- Forget and reconnect: If already listed but not connecting, select the headphones → Forget/Unpair, then pair them again.

- Restart both devices: Turn Bluetooth off/on and restart the phone and headphones.

- Check another device: Try connecting the headphones to another phone to determine whether the problem is with the headphones or the original phone.

- Final check: If they still do not connect, reset the headphones according to their instructions and try pairing again.

Indicators checked: Bluetooth ON/OFF status, headphone battery, pairing light, and device name in the Bluetooth list.

- External Webcam Not Detected in Zoom – Windows 10:

Checklist:

- Check USB connection: Disconnect and reconnect the webcam and try another USB port.

- Check Device Manager: Open Device Manager → Cameras/Imaging Devices and check whether the webcam is listed. Update or reinstall its driver if necessary.

- Check Windows privacy settings: Go to Settings → Privacy → Camera and make sure Allow access to the camera on this device is ON.

- Allow app access: Enable Allow apps to access your camera and make sure desktop apps are allowed to use the camera.

- Check Zoom settings: Open Zoom → Settings → Video and select the correct external webcam from the Camera dropdown.

- Close other apps: Close applications such as Camera, Teams, or Skype that may already be using the webcam, then restart Zoom.

4. Printer Showing “Offline” – Using Windows Troubleshooter:

Steps:

- Make sure the printer is powered ON and properly connected through USB or Wi-Fi.

- Open Start → Settings → Devices → Printers & scanners.

- Select the printer that shows Offline.

- Click Manage and select Run the troubleshooter (Windows 10 may show troubleshooting options through the printer settings).

- Windows will automatically check common problems such as:

- Printer connection

- Driver problems

- Printer configuration

- Print spooler issues

- Default printer settings

- Network/communication problems

- Follow the recommended fixes provided by Windows.

- After troubleshooting, print a test page to check whether the printer is working.

Information provided by the Troubleshooter: It identifies detected problems, shows the status of the printer and related services/settings, and may automatically apply or recommend a suitable fix